AVIONICS TASK ROUND 2

Task 3: Propulsion System

Consolidated Assignment, Knowledge Sources and Solution

# Table of Contents

1. Inputs	3

1.1 Mission Statement	3

1.2 Relevant Assignment Instructions	3

1.3 Task 3 Requirement	3

1.4 Static Hardware Inputs	3

1.5 Hard Performance Requirements	4

2. Expected Outputs	4

3. Glossary of Important Terms	4

4. Knowledge Sources and Elaboration	5

4.1 Assignment-Specific Hardware Sources	5

4.2 Common Knowledge Sources Supplied with the Assignment	5

4.3 Supplementary Engineering Sources	5

4.4 Source-Based Engineering Baselines	6

5. Relationship Between Tasks 1, 2 and 3	6

6. Solution	6

6.1 Corrected Structural Weight Budget	6

6.2 Battery and ESC Selection	7

6.3 Electrical Load Budget	7

6.4 Hover-Power and Endurance Calculation	8

Step A: Required Hover Thrust per Motor	8

Step B: Propulsion Power Estimate	8

Step C: Total Hover Power and Current	8

Step D: Usable Battery Capacity	8

Step E: Estimated Endurance	8

6.5 Technical Justification	8

6.6 Current, Power and Endurance Summary	8

6.7 Bonus: Alternative Motor Candidate	9

7. Final Engineering Note	9

# 1. Inputs

## 1.1 Mission Statement

Aeolus, the Greek god of wind, gifts Odysseus a bag containing all adverse winds to ensure a safe voyage home to Ithaca. However, Odysseus’ crew, suspecting that the bag contains gold, open it while he is asleep, unleashing a storm that blows them back out into the treacherous ocean and back to Aeolia. (this is actually from mythology btw)

Aeolus, the keeper of the winds, refused to help them again after this, but we at Team Aeolus can’t just leave our guy Odysseus out there in the sea to fend for himself. We must send out an autonomous drone for a search and rescue mission. The drone can’t weigh more than 2.5 kg. We have chosen a 10-inch frame for this mission.

Figure 1. Aeolus and Odysseus - mission context illustration.

## 1.2 Relevant Assignment Instructions

Wherever power or endurance is calculated, show the calculation together with the total current and power drawn.

All bonus questions are additional 10 marks.

Task 1: 10 marks; Task 2: 15 marks; Task 3: 20 marks.

## 1.3 Task 3 Requirement

Build a 10-inch drone.

Use Holybro S500 V2 2216-920KV motors.

Use 1045 (10 x 4.5 inch) propellers.

Select the best battery and ESC combination.

Keep endurance above 12 minutes.

Keep total takeoff weight below 2.5 kg.

Show battery calculations and justify the ESC selection.

Bonus: identify an alternative motor offering better thrust efficiency / lower power demand.

## 1.4 Static Hardware Inputs

| Input | Assignment Specification |
| --- | --- |
| Frame | 10-inch multirotor frame |
| Motor | Holybro S500 V2 Motor 2216-920KV |
| Propeller | 1045 / 10 x 4.5 inch propeller |
| Aircraft mass limit | 2.5 kg maximum takeoff weight |
| Endurance target | Greater than 12 minutes |

## 1.5 Hard Performance Requirements

MTOW < 2,500 g.

Estimated flight endurance > 12 minutes.

Battery voltage and capacity must be compatible with the motor/ESC system.

ESC continuous-current rating must be appropriate for the selected motor/propeller combination.

# 2. Expected Outputs

Battery configuration: cell count, nominal voltage and capacity.

Battery sizing calculation showing usable capacity and estimated endurance.

Exact ESC selection with current-rating justification.

Total hover-current and power estimate, including onboard avionics/communication loads.

Weight budget demonstrating compliance with the 2.5 kg MTOW limit.

Bonus alternative motor recommendation supported by published test data where available.

# 3. Glossary of Important Terms

- KV Rating (RPM/V): Motor speed constant describing approximate no-load revolutions per minute per applied volt. It is not a direct measure of torque or efficiency.

- 1045 Propeller: A propeller approximately 10 inches in diameter with 4.5 inches of nominal pitch.

- ESC (Electronic Speed Controller): Power electronic device that commutates the brushless motor and regulates motor speed/torque according to the flight-controller command.

- Continuous Current Rating: Current an ESC or conductor can carry continuously under specified cooling conditions without exceeding its thermal limits.

- Burst Current Rating: Higher current permitted only for a short specified duration. It must not be treated as a continuous operating rating.

- LiPo Cell Count (S): Number of battery cells connected in series. A 4S LiPo is 14.8 V nominal and 16.8 V fully charged.

- Battery Capacity (mAh / Ah): Electrical charge stored by the battery. A 5200 mAh battery has a nominal capacity of 5.2 Ah.

- C-Rating: Manufacturer discharge-rate rating. Continuous and burst C-ratings may be specified separately; theoretical current capability is capacity in Ah multiplied by the applicable C-rating.

- Depth of Discharge (DoD): Fraction of nominal battery capacity assumed usable before reaching the chosen safe landing/cutoff reserve.

- Hover Thrust: Thrust required to balance aircraft weight in steady hover. For a symmetric quadcopter it is approximately one quarter of total aircraft weight per motor.

- Thrust Efficiency (g/W): Static thrust produced per watt of electrical input under a specific motor, propeller, voltage and throttle test condition.

- MTOW: Maximum Takeoff Weight: total mass of the complete aircraft, including battery, avionics, payload, wiring and mounting hardware.

- Power Budget: Estimate of electrical power required by propulsion and non-propulsion loads during a defined operating condition.

- Engineering Margin: Additional allowance included because the exact vehicle differs from bench-test conditions and because wiring, regulators, propellers, airflow and payload vary in practice.

# 4. Knowledge Sources and Elaboration

The sources below are separated into (a) links explicitly supplied in the Task 3 requirement, (b) common learning resources supplied for all three avionics tasks, and (c) supplementary engineering sources used only where the supplied material does not provide the quantitative data required for battery, weight, current and endurance calculations.

## 4.1 Assignment-Specific Hardware Sources

https://zbotic.in/product/holybro-s500-v2-motor-2216-920kv-ccw/

The Zbotic motor page identifies the specified Holybro S500 V2 2216-920KV CCW motor and provides the key selection data used here: 920KV rating, 63 g motor mass, 3-4S LiPo compatibility, 22 mm stator diameter, 28 mm motor diameter and compatibility with 10-inch propellers. The page does not provide a thrust-versus-current test curve for the exact 1045 propeller, so it is not used to invent hover-current or efficiency values.

https://robu.in/product/orange-hd-propellers-104510x4-5-abs-dji-black-1cw1ccw-1pair-premium-quality/

The Robu link identifies the assignment propeller as the Pro-Range/Orange HD 1045 propeller, i.e. a 10 x 4.5 inch propeller. The supplied page is used to establish the required propeller product and geometry; it is not treated as a source of thrust, current or efficiency data because those performance curves are not provided in the assignment material.

## 4.2 Common Knowledge Sources Supplied with the Assignment

https://www.instructables.com/The-Ultimate-Guide-to-Building-a-Quadcopter-From-S/

https://www.youtube.com/watch?v=0xnZqpyrJRo

https://www.youtube.com/watch?v=u-LfLW9Kgis

https://www.youtube.com/watch?v=N_XneaFmOmU

These common resources provide the foundational quadcopter context used across Tasks 1, 2 and 3. They describe the basic aircraft hardware arrangement, including the battery, flight controller, ESCs, motors, camera system and video-transmission hardware, and show how added payload hardware affects aircraft mass and endurance. The supplied video material also provides a useful reference example of a 10-inch-propeller camera-drone configuration with a total mass of about 1.36 kg and approximately 12-13 minutes of flight time. This is used only as a contextual benchmark, not as proof of endurance for the present design because the payload, battery and operating conditions differ.

The common source material further explains that rotor speed produces thrust and that large propellers and propulsion hardware can introduce vibration into the airframe. These principles support the need to account for propulsion loading, component mass and electrical power carefully. They do not provide the exact Holybro 2216-920KV + Orange 1045 thrust/current data required for a precise endurance calculation.

## 4.3 Supplementary Engineering Sources

The following sources are used only to fill quantitative gaps that are not covered by the assignment-provided links. They support the corrected weight budget, battery selection, auxiliary-load estimate and propulsion proxy calculation.

https://holybro.com/products/s500-v2-kit

https://holybro.com/products/s500-v2-development-kit

https://holybro.com/products/pixhawk-6c-mini

https://docs.holybro.com/power-module-and-pdb/power-module-comparison

https://holybro.com/products/microhard-telemetry-radio-v2

https://docs.luxonis.com/hardware/products/OAK-D%20Lite

https://checkout.luxonis.com/products/oak-d-lite-1

https://www.raspberrypi.com/news/introducing-raspberry-pi-5/

https://www.radiomasterrc.com/products/rp3-expresslrs-2-4ghz-nano-receiver

https://www.walksnailir.com/shows/95/29.html

https://genstattu.com/tattu-5200mah-14-8v-35c-4s1p-lipo-battery-pack-with-xt60-plug.html 292

https://store.tmotor.com/product/air-gear-450II-combo-set-no-esc.html

## 4.4 Source-Based Engineering Baselines

The assignment-specific Zbotic data confirms that each specified motor weighs 63 g, is rated at 920KV, supports 3-4S LiPo operation and is intended for 10-inch propellers. The Robu requirement fixes the propeller geometry at 1045 (10 x 4.5 inches). These are treated as the primary hardware constraints for Task 3.

Supplementary Holybro data lists the S500 V2 ARF kit at approximately 782 g including the frame, four 2216 KV920 motors, four BLHeli_S 20 A ESCs, built-in PDB and 1045 propellers. These items must therefore not be added again as separate masses when the 782 g kit value is used.

Holybro recommends a 4S 3000-5000 mAh 20C+ battery for the S500 V2 and reports approximately 18 minutes of hover with a 5000 mAh battery and no additional payload on the S500 V2 development kit. This manufacturer result is used only as a sanity check because the autonomous search-and-rescue configuration carries additional compute, sensing and communication hardware.

The exact Holybro 2216-920KV + Orange 1045 thrust/current curve is not supplied by the assignment links. For the endurance estimate below, published T-Motor AIR2216II-KV920 + T1045 4S bench data is used only as an engineering proxy because it matches the 2216 stator class, 920KV rating and 10 x 4.5 propeller size. A 15% propulsion margin is then added. Final acceptance must use measured current from the actual Holybro/Orange combination.

# 5. Relationship Between Tasks 1, 2 and 3

Task 3 closes the system-level design loop. Task 1 selected the OAK-D Lite + Raspberry Pi 5 sensing/compute architecture, while Task 2 selected the RC, telemetry and video communication hardware. Those airborne components add mass and electrical load, so they must be included in the Task 3 weight and endurance budgets.

The connection should be treated quantitatively rather than as a fixed “payload penalty.” Component mass is taken from manufacturer data where available, while uncertain mounting, cooling, regulator and wiring mass is carried as an explicit engineering allowance. Similarly, avionics power is estimated using published limits plus conservative design allowances and should be verified by bench measurement before flight.

# 6. Solution

## 6.1 Corrected Structural Weight Budget

The original calculation double-counted motors, ESCs and propellers because the 782 g Holybro S500 V2 ARF figure already includes those items. The corrected budget starts from the complete ARF mass and adds only hardware that is not already included.

| Component | Mass (g) | Basis |
| --- | --- | --- |
| Holybro S500 V2 ARF kit | 782.0 | Manufacturer figure; includes frame, 4 motors, 4 x 20 A ESCs, PDB and 1045 propellers |
| Pixhawk 6C Mini | 42.4 | Current Model A reference mass |
| PM02 V3 power module | 20.0 | Manufacturer mass |
| Holybro M10 GPS | 32.0 | Manufacturer mass |
| Raspberry Pi 5 | ~46 | Typical board-only mass; final build should be weighed |
| OAK-D Lite | 61.0 | Manufacturer mass |
| RadioMaster RP3 receiver | 4.6 | Including two antennas |
| Holybro Microhard P900 V2 airborne radio | 81.0 | Including antenna |
| Walksnail Avatar V2 VTX + camera + 2 antennas | ~26.6 | 15.4 g VTX + 7.2 g camera + 2 x 2 g antennas |
| Regulators, cooling, mounts, wiring and connectors | 100.0 | Conservative engineering allowance |
| Estimated dry mass | ~1,196 | Sum before flight battery |
| Tattu 4S 5200 mAh 35C battery | 436.5 | Manufacturer net weight |
| Estimated MTOW | ~1,633 | Well below the 2,500 g assignment ceiling |

Mass margin to the assignment ceiling is approximately 2,500 - 1,633 = 867 g. This is not a recommendation to use the full remaining mass; additional battery mass would also increase required hover thrust and propulsion power.

## 6.2 Battery and ESC Selection

| Item | Selected Hardware | Justification |
| --- | --- | --- |
| Battery | Tattu / Gens Ace 4S 5200 mAh 35C LiPo | 4S is consistent with the Holybro S500 V2 recommendation; 5200 mAh provides slightly more capacity than the manufacturer 5000 mAh benchmark while remaining light (~436.5 g). |
| ESC | Holybro BLHeli_S 20 A ESC x4 | This is the manufacturer-selected ESC supplied with the S500 V2 2216 KV920 motor / 1045 propeller combination. Using the validated pairing is stronger than relying on an unsupported assumed peak-current number. |

The 35C continuous battery rating corresponds to a theoretical current capability of 5.2 Ah x 35 = 182 A, far above the estimated hover current. In practice, ESC, PDB, wiring, connectors, thermal limits and voltage sag are more relevant constraints than the battery C-rating alone.

## 6.3 Electrical Load Budget

A single unsupported “12 W avionics load” is not used. The following conservative design budget is used instead. Values marked as allowances should be replaced by measured input power during final bench testing.

| Load | Design Power | Basis |
| --- | --- | --- |
| Raspberry Pi 5 | 12.0 W | Published worst-case/peak board consumption is around 12 W for intensive workloads. |
| OAK-D Lite | 5.0 W | Upper design value from base streaming plus AI, stereo and encoder subsystems. |
| Microhard P900 V2 | 20.7 W | Conservative worst-case: 1.4 A transmit-current figure multiplied by 14.8 V nominal 4S bus. Actual power depends on supply voltage and transmit duty cycle. |
| Walksnail Avatar V2 video system | 10.0 W | Engineering allowance; manufacturer page specifies voltage range and RF output but not a complete electrical input-power figure. |
| Pixhawk + GPS + RP3 receiver | 3.0 W | Engineering allowance covering controller, GNSS and RC receiver. |
| DC-DC conversion / miscellaneous losses | 4.0 W | Engineering allowance for regulator and wiring losses. |
| Total auxiliary electrical budget | 54.7 W | Conservative design value used for endurance sizing. |

## 6.4 Hover-Power and Endurance Calculation

Step A: Required Hover Thrust per Motor

Estimated MTOW = 1,633 g

Required hover thrust per motor = 1,633 / 4 = 408 g per motor

Step B: Propulsion Power Estimate

T-Motor publishes 4S test points for an AIR2216II-KV920 with a T1045 propeller: 373 g at 46 W and    447 g at 58 W. Linear interpolation around the required 408 g hover point gives approximately 51.7 W per motor.

Proxy propulsion power = 51.7 W x 4 = 206.8 W

Applying a 15% engineering margin for the different motor/propeller, installation and flight conditions:

Design propulsion power = 206.8 W x 1.15 = 237.8 W

Step C: Total Hover Power and Current

Total design hover power = 237.8 W propulsion + 54.7 W auxiliary = 292.5 W

At 4S nominal voltage (14.8 V):

Estimated average hover current = 292.5 / 14.8 = 19.8 A

Step D: Usable Battery Capacity

Nominal capacity = 5.2 Ah

Using an 80% usable-capacity assumption: usable capacity = 5.2 x 0.80 = 4.16 Ah

Step E: Estimated Endurance

Estimated hover endurance = (4.16 Ah x 60) / 19.8 A = approximately 12.6 minutes

The maximum average current that still permits exactly 12 minutes at the same 80% usable-capacity assumption is 4.16 Ah / 0.20 h = 20.8 A. The conservative design estimate of 19.8 A is therefore inside the requirement, but with limited margin. Bench and flight validation are essential.

## 6.5 Technical Justification

Why 4S: Holybro recommends a 4S 3000-5000 mAh battery for the S500 V2. A 4S pack is therefore the most direct manufacturer-aligned choice for the specified 2216 KV920 motors and 1045 propellers. Higher bus voltage can reduce current for a given electrical power, but actual motor current must come from the real motor/propeller operating point rather than a simple voltage-only assumption.

Why 5200 mAh: The 5200 mAh pack provides 4.16 Ah of usable capacity under the conservative 80% assumption, weighs about 436.5 g, and keeps the estimated aircraft mass far below the 2.5 kg ceiling. The endurance calculation gives approximately 12.6 minutes under the deliberately conservative power budget.

Why 20 A ESC: The Holybro S500 V2 manufacturer configuration pairs the 2216 KV920 motors and 1045 propellers with BLHeli_S 20 A ESCs. This direct manufacturer pairing is the primary justification. Sustained current, ESC temperature and wiring temperature should still be verified under the final propeller, battery and payload configuration.

Validation requirement: Because the exact Holybro motor/Orange propeller thrust-current curve is not available, the final design should be bench-tested with a wattmeter or power analyzer. Measure total current at hover-equivalent thrust and then validate actual flight endurance with the complete payload installed.

## 6.6 Current, Power and Endurance Summary

| Parameter | Corrected Design Value |
| --- | --- |
| Battery | 4S 5200 mAh 35C LiPo |
| Battery nominal voltage | 14.8 V |
| Battery mass | ~436.5 g |
| Estimated MTOW | ~1.63 kg |
| Required hover thrust | ~408 g per motor |
| Design propulsion power | ~237.8 W including 15% margin |
| Auxiliary electrical budget | ~54.7 W |
| Total hover power | ~292.5 W |
| Estimated hover current | ~19.8 A |
| Usable capacity assumption | 80% = 4.16 Ah |
| Estimated hover endurance | ~12.6 minutes |
| Assignment requirement | >12 minutes |
| ESC | Holybro BLHeli_S 20 A x4 |

## 6.7 Bonus: Alternative Motor Candidate

A defensible alternative is the T-Motor AIR2216II-KV920 used with the manufacturer-matched T1045 propeller. It is not claimed here to be definitively more efficient than the Holybro motor because equivalent Holybro bench data is not available. Its advantage for the bonus analysis is that T-Motor publishes a complete 4S thrust/current/power table for the same 2216 / 920KV / 10 x 4.5 class.

| Published T-Motor Point | Thrust | Power | Efficiency |
| --- | --- | --- | --- |
| 45% throttle at 16 V | 373 g | 46 W | 8.15 g/W |
| 50% throttle at 16 V | 447 g | 58 W | 7.76 g/W |
| 100% throttle at 16 V | 1,332 g | 260 W | 5.13 g/W |

At the present aircraft weight, the required 408 g/motor hover point falls between the 45% and 50% published test points. This motor is therefore a suitable evidence-based candidate for A/B testing. To satisfy the bonus claim rigorously, both the Holybro and candidate motor should be tested on the same 4S battery and the same 1045 propeller, and thrust-per-watt should be compared at the same required thrust.

# 7. Final Engineering Note

The revised calculation deliberately separates manufacturer facts from engineering estimates. The 4S/5200 mAh/20 A architecture meets the >12 minute requirement in the conservative analytical model, but the result is close enough to the limit that measured hover current should be treated as the acceptance criterion before submission of a physical build.